Dear sir or madam,

I am submitting my written evidence as a response to the call for evidence: critical minerals to the Business and Trade Sub Committee on Economic Security, Arms and Export Controls.

My name is Prof. Jose Gonzalez-Rodriguez, and I work at the University of Lincoln. I am also the founder of Environmental Selective Extractions (ESE), a specialist research, design and manufacturing venture dedicated to recycling and extraction of valuable/hazardous compounds. I am submitting this evidence as an individual.

My research focuses on analytical chemistry, materials recovery, and the development of novel technologies for the extraction of critical elements.  I have developed two patent applications for recycling critical compounds of interest for industrial applications.

I learned of the call for evidence for this committee through the UK Parliament Committees website.

Having read the terms of reference, I would like to comment on some of the aspects covered in the remit of the enquiry. I have a special interest in the use of new technologies in recycling, and these may have the key to some solutions to the domestic supply chain questions posed in this enquiry.

Summary of conclusions

- The need to establish a comprehensive national analytical programme to map critical minerals in industrial waste led by government agencies.

- Recycling is not yet treated as a core strategic pillar. In practice, it remains underdeveloped despite its potential to reduce dependence on imports.

- The need to increase targeted government funding for novel extraction and recycling technologies for low-yield sources, bringing together research technology organisations (RTOs) and industrial partners.

- The need for increased government investment in innovation and scale-up.

- The need to develop domestic midstream processing capabilities alongside extraction.

The domestic supply chain

The UK’s national priorities are closely tied to a group of minerals essential for energy transition, defence, and advanced manufacturing. These include lithium, cobalt, rare earth elements, vanadium, and gallium, among others. A central vulnerability lies not only in access to raw materials but in the concentration of midstream processing capacity in a limited number of countries. Even where raw materials are available globally, refining and processing are often geographically concentrated, creating a systemic risk. I have highlighted this issue previously 1 where the technological supply depends on a small number of actors controlling key stages of production. In practical terms, this means that restrictions, whether geopolitical or economic, can propagate rapidly through multiple industrial sectors. The present Iran crisis offers a clear example where dependence on trade and imports can seriously affect the economy and resilience of a country like ours. The UK faces a structural vulnerability as an island economy. Transport dependencies and limited domestic extraction increase exposure to supply shocks. Moreover, the lack of adequate and extensive manufacturing and transformative industries to process these metals adds to our increasing vulnerability and dependencies.

Looking ahead, demand for critical minerals is expected to grow significantly over the next 10 to 20 years, driven by electrification, renewable energy systems, battery technologies and advanced electronics. These would likely intensify competition for resources and further expose weaknesses in our current supply chains.

A major constraint on UK resilience is the lack of a comprehensive understanding of domestic secondary-source resources, particularly those contained in industrial residue waste streams and by-products. In a previous submission [1], I highlighted the need for an analytical survey of critical materials, including the identification of matrix concentrations and temporal variation. Without these data, it is not possible to assess the contribution these sources could make to the national supply. Valuable elements can be found in low concentrations across a very diverse range of industrial waste streams in the UK. While individually small, collectively these sources may provide a meaningful contribution to supply resilience, particularly in crisis scenarios.

Traditionally, extraction technologies are often unsuitable for low-grade or complex sources. Processes such as hydrometallurgy or pyrometallurgy typically require relatively high concentrations to be economically viable. As a result, significant quantities of potentially valuable materials present in the waste remain unexploited. Emerging technologies, including selective polymeric extraction, ionic liquids and deep eutectic solvents, offer promising alternatives. These approaches can enable selective recovery from low deal sources, but they remain at relatively early stages of development. The UK has strong capabilities in this area, both in academia and industry. However, further investment is required to scale these technologies and bring them to operational readiness.

A case study of how valuable these waste streams can be is found in the collaboration between industrial partners, GSA Environmental Ltd, and my group at the University of Lincoln. Together, we developed a novel technology combining molecularly imprinted polymers and traditional hydrometallurgy to extract Scandium and Yttrium from UK-produced titanium dioxide (TiO2) waste. This work, funded by the British Government through the Innovate UK (IUK) CLIMATES programme, demonstrated both the viability of this approach and the economic value of the residues generated in the TiO2 industry. It also serves as a clear example of how targeted government investment can make a difference.

Our analytical results for the UK-produced titanium dioxide waste indicated not only an appreciable scandium and Yttrium content, but also the presence of other valuable critical elements such as neodymium, dysprosium and hafnium. Higher concentrations of other metals, such as iron, aluminium, manganese or chromium (to cite some) were also found. Using the average concentrations measured in the samples we analysed, one tonne of this material contains approximately 48g of scandium, 60 g of yttrium, 37 g of neodymium, 15 g of dysprosium and one gram of hafnium, giving a combined indicative value of roughly $514.00 per tonne of waste (current prices). This value can be increased by the recovery of other common metals, as mentioned.

By comparison, typical gold ores processed commercially contain between 1-5 grams per tonne of gold, corresponding to approximately $70 to  $350 per tonne at current prices.

There is no single official published figure for total UK TiO2 waste generation, but a good estimate can be obtained by using TiO2 production (230,000 tonnes per annum) and typical waste generation rates of 1.5-5 tonnes of residue per tonne of product [2]. The TiO2 waste stream produced in the UK represents a clear example of an underutilised secondary resource for critical minerals. Based on the previous calculations, the theoretical value of this waste stream lies in the range of approximately $175 to $575 million per year, before accounting for recovery efficiencies, processing costs and market factors.

This is a clear example of how an analytical survey of industrial waste in a specific sector can offer valuable insight to evaluate and develop an analytical extraction strategy on what elements are useful for midstream industries.

A number of industrial waste streams already present in the UK contain measurable concentrations of critical minerals and are generated at a significant scale. It is difficult to estimate an exact amount of the waste, but cold combustion residues (~257,000 tonnes per year), steelmaking slugs (~ 770,000 tonnes per year), and blast furnace slags (> 1.2 million tonnes per year) are produced across major industrial clusters in Northumberland, Yorkshire, South Wales and Northern England.  Analytical data obtained from the literature for similar waste indicate that these may contain rare earth elements and other critical metals in concentrations ranging from tens to several hundred parts per million. Taken together, these figures demonstrate that the UK already possesses a geographically distributed and industrially accessible resource based on secondary materials which, if systematically characterised and coupled with appropriate extraction technologies, could contribute meaningfully to supply chain resilience. These are only examples of some well-known industries presenting critical elements in their waste, and a systematic analytical survey and review of other industries may open the door for further sources.

Beyond the technological challenges, there are several additional economic and regulatory barriers that limit domestic production.  Planning and permitting processes can delay projects significantly and hinder progress. Economic competitiveness is particularly important when compared to lower-cost international producers. In this case, national security needs to favour domestic production to avoid overreliance on external suppliers. Importantly, there is a skills shortage in specialised areas in extraction and materials processing that needs to be corrected. There is also a fragmentation of the support mechanisms to develop and scale up production, which makes it difficult for companies to access funding and coordinated help.

Prof. Jose Gonzalez-Rodriguez

Founder of Environmental Selective Extractions (ESE) and Professor at the University of Lincoln

April 2026

References

- Written evidence submitted by Dr Jose Gonzalez-Rodriguez (MIN0018) to the Foreign Affairs Committee inquiry into critical minerals (2023). committees.parliament.uk/writtenevidence/118790/html/

- How to make TiO2 manufacturing more efficient. GSA environmental website. Courtney Farrow (2020). https://www.gsa-environmental.com/2020/10/26/how-to-make-tio2-manufacturing-more-efficient/